| Name / Capacity | Type | Volume | Weight / metric ton (tonne) = 1000 kg | Tare weight = Empty Weight | Gross Weight = Total |
| --- | --- | --- | --- | --- | --- |
| 20’ TEU | Standard Container | 33 m3 | 28 – 29 (depending on the specific container and regulations) |  |  |
| 40’ FEU | Standard Container | 67-68 m3 | 26 – 28 (depending on the specific container and regulations) |  |  |
| High Cube Containers | Container | 37 m3 (more volume) | less weight capacity due to the increased height |  |  |
| 20’ TEU DV | Dry Van Container | 33 m3 | 28 – 29 |  |  |
| EUR Pallet | Pallet | 800 x 1,200 mm | 1 – 1.5 (static load) 0.5 – 1 (dynamic load) |  |  |
| North American Pallet | Pallet | 48 x 40 inches | 2 – 2.5 (static load) 1.5 – 2 (dynamic load) |  |  |
| Trailer | Trailer |  | 26 (24 in EU) |  |  |
| Car & Trailer |  |  | 40 – 50 |  | 60 – 74 |
| HCT Vehicle (High Capacity Transport) |  |  |  |  |  |
| Duo-Trailer | Trailer | 200 m3 | 50 | 30 | 80 |
|  |  | 165 m3 | 36 | 30 | 66 |
| Standard Truck |  |  |  |  | 40 – 44 |
| EMS-Truck |  |  |  |  | 60 – 74 |
| Light Truck | Light Truck |  | 1 |  | 3.5 |
| Pickup Trucks Delivery Vans Utility Trucks | Light Truck |  | 0.45 – 1.36 (smaller compact or midsize) 0.68 – 1.814 (full size) |  |  |
| Cargo Vans | Light Truck |  | 0.68 – 1.814 |  |  |
| SUVs and Crossover Vehicles | Light Truck |  | 0.363 – 0.68 |  |  |
| Heavy Truck | Truck - 6 pll |  | 7.6 |  | 23 |

Gross weight is the total weight of the loaded container along with the goods inside.

Tare weight is the weight of the empty container itself without any cargo.

Net weight is calculated by subtracting the tare weight from the gross weight, giving the weight of the goods being transported.

Pll = pallets

The form of logistics

- Production logistics

- Distribution logistics

- Third part logistics / 3pl

- Fourth party logistics / 4pl

Load Carrier

A load carrier typically refers to any vehicle, equipment, or system designed to transport goods, materials, or items from one place to another.

Load carriers come in various forms, from simple tools like containers, trailers, carts, trolleys, and pallets to more complex machinery like forklifts, trucks, and conveyor belts.

Load carriers are utilized across industries such as manufacturing, logistics, warehouses, agriculture, and transportation to efficiently move, store, and transport goods. The choice of a load carrier depends on the type of goods being transported, the distance, the weight and size of the load, and the specific requirements of the tasks.

Requirements for unit load:

- High concentration of goods flow

- The flow of goods must be regular.

- The logistics chain can contain several means of transport.

Standardized load carriers can be used intermodally by sea, rail, or road:

- Container

  - Standardized metal boxes used for transporting goods internationally. The most common sizes are 20 feet (TEU – Twenty-Foot Equivalent Unit) and 40 feet (FEU – Forty-Foot Equivalent Unit). Containers allow for easy handling, stacking, and transfer between different transport modes without the need to unpack and repack the goods. Containers ensure security, protect goods from damage, and streamline the loading and unloading processes.

- Trucks and vans – Various types and sizes used for transporting goods over short or long distances.

- Trailer

  - Trailers are designed to be pulled by trucks or others motor vehicles. They come in various sizes and configurations, such as flatbed trailers, refrigerated trailers (reefers), dry vans, and specialized trailers for specific cargo types. Trailers are widely used for land transportation, especially for short or regional hauls, and can be interchanged among different trucks or tractor units.

- Pallets

  - Flat structures made of wood, plastic, or metal used to stack goods for transportation and storage. The most common sizes are the EUR pallet (800x1200 mm) and the North American pallet (48x40 inches). Pallets are compatible with forklifts and pallet jacks, facilitating efficient loading and unloading.

- ISO Tanks

  - Standardized tanks used for transporting bulk liquids, chemicals, or gases. They comply with ISO (International Organization for Standardization) standards for easy handling and compatibility across transport modes.

- Forklifts

  - Industrial vehicles used for lifting and transporting heavy loads within warehouses or storage facilities.

  - Smaller forklifts have capacities of 3,000 to 5,000 pounds.

  - Larger forklifts have capacities of 20,000 to 100,000 pounds

- Conveyor belts and rollers

  - Automated systems used in factories or distribution centers for moving items along a specific path.

- Carts and trolleys

  - Manual or powered devices used for moving smaller loads within a confined area.

- HCT Vehicles (High Capacity Transport)

  - Specialized transport vehicles or transport systems designed to carry larger and heavier loads than standard vehicles. The primary goal is to increase the capacity of freight transportation, reducing the number of trips required to transport goods, which can potentially lead to increased efficiency and reduced environmental impact.

  - Increased Length – Longer than standard trucks, accommodate more cargo

  - Additional Axles – Extra axles to distribute the weight of the load more evenly.

  - Increased Load Capacity – Carry larger volumes or heavier loads compared to conventional transport vehicles, such as trucks or trailers. Aim to optimize the use pf transport infrastructure and reduce overall transport costs.

  - Efficiency and Productivity – Result in reduced fuel consumption, fewer emissions, decreased traffic congestion, and potentially lower transportation costs per unit of cargo.

  - Regulatory Considerations – Initiatives often require specialized regulations and guidelines due to the increased size, weight, and potential impact on road safety and infrastructure. Regulations vary by region and country and typically involve restrictions on routes, vehicle dimensions, weight distribution, and safety measures.

  - Environmental Impact – Reducing the number of trips needed to transport goods, have the potential to lower the carbon footprint associated with freight transportation.

- Duo-Trailer

  - Two Trailers – Consist of two separate trailers connected in tandem and towed by a single tractor unit.

  - Increased Load Capacity – Transportation of larger volumes of goods in a single trip.

  - Articulation Mechanism – Trailers are commonly connected via an articulation mechanism that allows for flexibility and maneuverability while maintaining stability during travel.

  - Regulatory Considerations – Specific regulations and guidelines imposed by transportation authorities. Regulations often address issues related to road safety, weight distribution, length restrictions, and maneuvering capabilities.

- EMS-Vehicle (Emergency Medical Services)

- Double-Decker Trailer

  - Two Levels – The trailer is divided into two levels or decks, commonly connected by internal ramps or lifts. This design doubles the cargo space compared to a single-level trailer.

- Distribution Vehicle

  - Transporting goods or products from distribution centers or warehouses to various destinations, such as retail stores, businesses, or directly to consumers. These vehicles are essential components of the supply chain and logistics operations, facilitating the efficient distribution of goods to their intended locations.

- Light Vehicles

  - For personal use, transportation, or light-duty commercial purposes.

  - Smaller in size, lower gross vehicle weight ratings (GVWR), transporting fewer passengers or lighter loads compared to heavy-duty vehicles.

  - Passenger Cars

  - Light Trucks and SUVs

  - Lower Gross Vehicle Weight

  - Fuel Efficiency

- Heavy Vehicles

  - Transporting substantial loads, carrying  large number of passengers, or performing heavy-duty tasks.

  - Higher gross vehicle weight ratings (GVWR), handling larger payloads or heavier loads compared to light vehicles.

  - Trucks and Commercial Vehicles

  - Higher Weight Ratings

  - Specialized Configurations

  - Usage in Various Industries

  - Regulations and Licensing

- Light Trucks

  - Carrying cargo or performing light-duty commercial tasks.

  - Offer a balance between passenger accommodation and cargo-carrying capabilities, making them suitable for various purposes, including personal use, commercial applications, and small-scale businesses.

  - Cargo Capacity – Allow for carrying moderate to substantial cargo loads. Often have a designated cargo area separate from the passenger compartment, providing space for transporting goods, equipment, or tools.

  - Versatility – Used for personal transportation, small-scale commercial operations, deliveries, construction, landscaping, and other light-duty tasks.

  - Variety of Configurations – Pickup trucks, delivery vans, utility trucks, and small commercial trucks. Have different bed sizes, cab styles, and cargo-carrying options.

  - Towing Capabilities – Equip with towing capabilities, allow to haul trailers or other equipment.

  - Potential for Customization – Offer potential for customization with accessories or modifications to suit specific business or personal needs, such as tool racks, specialized compartments, or bed covers.

- Heavy Trucks

  - Transporting substantial loads, performing heavy-duty tasks, and meeting the demands of commercial or industrial applications.

  - Handling large payloads, towing heavy trailers, and operating in various industries that require robust transportation solutions.

  - High Payload Capacity – Build with robust chassis, powerful engines, and sturdy suspension systems capable of carrying large and heavy loads. Higher gross vehicle ratings compared to light trucks or passenger vehicles.

  - Versatility – Specialized forms to suit different purposes, such as flatbed trucks, dump trucks, tractor-trailers, tanker trucks, and heavy haulers used in construction, logistics, mining, and other industries.

  - Towing Capability – Haul trailers, heavy machinery, or other equipment necessary for industrial operations. May be equipped with powerful engines and towing accessories like fifth-wheel hitches or gooseneck couplings.

  - Size and Power – Larger and more powerful than light trucks, requiring specialized driving skills and often necessitating commercial driver’s licenses (CDL) for operation.

  - Industry-Specific Users – Utilized in various industries such as transportation and logistics for hauling freight, construction for transporting materials, agriculture for moving produce or equipment, and other applications that demand heavy-duty transportation solutions.

Parcels are generally smaller individual packages or small shipments that are light weight and manageable for manual handling. Weight ranges for parcels can vary by courier or postal service but typically range from 1 kg up to 30 kg.

Piece goods refer to larger individual items or goods that are not bulk or heavy enough to qualify as partial or full loads. Weight ranges for piece goods often gall between 30 kg and several hundred kg, depending on the specific transportation provider and their classification.

Partial loads, also known as LTL (Less-Than-Truckload) shipments, typically range from several hundred kg to a few thousand kg. Weight ranges for partial loads can vary significantly but generally fall between 500 and 10,000 kg.

Full loads refer to shipments that fill an entire transport unit, such as a truck or trailer, with one customer’s goods. Weight ranges for full loads can start from 10,000 and can go up to the maximum load capacity allowed by road transportation regulations, which could be tens of thousands of kg or more, depending on the vehicle type and local regulations.

Package < 100kg

General Cargo < 2500 kg

Party Goods > 2500 kg

Hellest, full car > 20,000 kg

Bulk Goods 2500 – 18000 kg

Tow Truck

- Advantages

  - Intermodal Compatibility – Enable seamless transfer and handling between different modes of transportation without the need for repacking or unloading cargo.

  - Efficiency – Standardized dimensions allow for better space utilization in transport vehicles and storage facilities, reducing wasted space and optimizing loading capacities.

  - Security and Protection – Standardized load carriers provide a level of protection against damage, theft, and adverse weather conditions during transit.

  - Cost-effectiveness – Standardization reduces handling costs, streamlines logistics operations, and enables economies of scale, resulting in overall cost savings.

Cargo Carrier Flight

A cargo carrier flight refers to an aviation operation specifically dedicated to transporting goods or freight rather than passengers. These flights are conducted by cargo airlines or commercial airlines that have dedicated cargo services.

Purposes:

- Freight Transport – These flights are primarily focused on transporting goods, merchandise, or cargo from one location to another. They handle a wide range of items, including perishable goods, machinery, electronics, pharmaceuticals, textiles, and more.

- Logistics and Supply Chain – Cargo carrier flights are crucial components of global supply chains, allowing companies to move goods quickly and efficiently across international borders. They contribute significantly to trade, manufacturing, and commerce by ensuring timely delivery of products.

- Specialized Cargo – Some cargo carrier flights handle specialized freight, such as oversized cargo, hazardous materials, live animals, perishable goods requiring temperature-controlled environments (like pharmaceuticals or fresh produce), and valuable items that require high-security transport.